# CECS 447 Project 2 - UART

By: Dr. Min He

Total: 20 points

Extra Challenge: 5 points

Preparation:

You will need two LaunchPads. You will use a serial terminal application, such as Tera term, Putty, realterm, etc., which can perform serial port communication on the PC.

Book Reading: 	Textbook Chapter 8, Sections 8.1, and 8.2

Reference Project:	UARTPC2B, UARTB2B

Starter Projects:	CECS447project2MCU1.c, CECS447project2MCU2.c.

Purpose:

In this lab you will learn how to set up and use UART modules on two TM4C123 Launchpads to establish a communication system shown below.

The system consists of four devices and three serial links: PC serial terminal 1↔MCU1, MCU2↔PC serial terminal 2 and MCU1 ↔ MCU2. Three different baud rates are required for the above three serail links. For serial link MCU1 ↔ MCU2, different UART modules are required for different microcontrollers. The following baud rate are not allowed in this project: 115200, 38400, and 57600. UART1, UART2 and UART3 are not allowed.

The system will work in one of the three different modes. A main menu will be provided on the PC serial terminal connected to MCU1 for a user to select a service mode.

UART 3       UART0

Serial Terminal 2

UART0            UART2

Serial Terminal 1

PC                                                  MCU1	                                           MCU2		                            PC

# The system will start with the following outputs on the two PC serial terminals. MCU1 will be the master in this communication system to initial data communication.

# Welcome to CECS 447 Project 2 – UART

# MCU1

# Main Menu

# PCMCU1 LED Control.

# MCU1MCU2 Color Wheel.

# PC1MCU1MCU2 PC2 Chat Room

Please choose a communication mode (enter 1 or 2 or 3):

# Welcome to CECS 447 Project 2 – UART

# MCU2

# Waiting for command from MCU1 …

PC Serial Terminal 1		                          PC Serial Terminal 2

# Once a user selects a communication mode, the system will stay in the selected mode until it exits the mode. A detailed description for each mode is given below.

# Mode 1: PC1MCU1 LED Control

# In this mode, only MCU1 and PC serial terminal 1 are involved in communication. MCU2 stays in idle state and PC serial terminal 2 displays the same starting message. PC serial terminal 1 will display Mode 1 menu. See both serial terminal displays below:

# Welcome to CECS 447 Project 2 – UART

# MCU2

# Waiting for command from MCU1 …

# Mode 1 Menu

# Please select an option from the following list (enter 1 or 2 or 3):

# Choose an LED color.

# Change the brightness of current LED(s).

# Exit.

PC Serial Terminal 1  						PC Serial Terminal 2

# Option 1: Once a user selects 1 from the above menu, MCU1 will initiate the communication by sending a prompt to PC serial terminal 1 for a user to select LED color to be displayed on MCU_1. Available colors are r(red), g(green), b(blue), y(yellow), c(cyan), p(purple), w(white), d(dark).

# Please select a color from the following list:

# d(dark), r(red), g(green), b(blue), y(yellow), c(cyan), p(purple), w(white)):

PC Serial Terminal 1

# After the user enters one of the above letters followed by a carriage return, MCU1 should change its LEDs to the specified color and send a confirmation message back to the PC serial terminal 1. Example confirmation message when ‘r’ is sent: “Red LED is on”. Then the Mode 1 menu will be displayed again on PC serial terminal 1.

# PC Serial Terminal 1

# Red LED is on.

# Mode 1 Menu

# Please select an option from the following list (enter 1 or 2 or 3):

# Choose an LED color.

# Change the brightness of current LED(s).

# Exit.

# Option 2: Once a user selects 2 from the Mode 1 Menu, the following prompt will be displayed on PC serial terminal 1:

# PC Serial Terminal 1

Please enter a decimal number from 0 to 100 followed by a return:

# After a user enters a decimal number, the number will be used to determine the duty cycle from 0% to 100%. A PWM signal will be generated based on the duty cycle and used to control the brightness of the current LED(s). A confirmation message will be sent to PC serial terminal 1 followed by Mode 1 Menu. See the following display for an example.

# PC Serial Terminal 1

Red LED is displayed at 30% brightness.

# Mode 1 Menu

# Please select an option from the following list (enter 1 or 2 or 3):

# Choose an LED color.

# Change the brightness of current LED(s).

# Exit.

# Option 3: Once a user selects 3 from the Mode 1 Menu, the system will exit Mode 1, and PC serial terminal 1 will display the main menu again.

# Mode 2: when in this mode, MCU1 will initiate communication. At any time in this mode, a ‘^’ symbol entered by a user at PC serial terminal 1 will end this mode. MCU1 UART0 RX interrupt will be used for a user to terminate this mode. Upon entering Mode 2, the two PC terminals will display the following messages:

# Mode 2 MCU1: press ^ to exit this mode

# In color Wheel State.

# Please press sw2 to go through the colors in the following color wheel: Dark, Red, Green, Blue, Yellow, Cran, Purple, White.

# Once a color is selected, press sw1 to send the color to MCU2.

# Current color: Dark

# Mode 2 MCU2

# Waiting for color code from MCU1 …

PC Serial Terminal 1		                          PC Serial Terminal 2

# The two microcontrollers will take turns changing their LED colors and send the selected color to the other microcontroller starting with MCU1.

# Step 1: Use sw2 on MCU1 to go through the colors in a color wheel in a round robin order. Every time the LED color is changed, update the current color display on Launchpad and PC serial terminal 1. Example:

# Current color: red

# Step 2: Once MCU1 selects a color and sends it to MCU2 by press sw1, MCU1 goes to idle state, waiting to receive color code from MCU2. The two PC serial terminals should update their display like the following:

# Mode 2 MCU1: press ^ to exit

# Current color: Red

# Waiting for color code from MCU2 …

# Mode 2 MCU2

# In color Wheel State.

# Please press sw2 to go through the colors in the following color wheel: Dark, Red, Green, Blue, Yellow, Cran, Purple, White.

# Once a color is selected, press sw1 to send the color to MCU1.

# Current color: Red

PC Serial Terminal 1		                          PC Serial Terminal 2

# Now it is MCU2’s turn to change color and send the selected color to MCU1.

# Step 3: Every time the LED color is changed on MCU2, update the current color display on Launchpad and on PC serial terminal 2.

# Step 4: Once MCU2 selects a color and sends it to MCU2 by pressing sw1, MCU2 goes to idle state, waiting to receive color code from MCU1.

# Mode 2 MCU1: press ^ to exit

# In color Wheel State.

# Please press sw2 to go through the colors in the following color wheel: Dark, Red, Green, Blue, Yellow, Cran, Purple, White.

# Once a color is selected, press sw1 to send the color to MCU2.

# Current color: Green

# Mode 2 MCU2

# Current color: Green

# Waiting for color code from MCU1 …

PC Serial Terminal 1		                          PC Serial Terminal 2

# Repeat Step 1 to 4. At any time in mode 2, a ‘^’ symbol sent from PC serial terminal 1 to MCU1 will terminate mode 2. MCU1 UART0 RX interrupt is required to capture this event.

# After Mode 2 is terminated, all LEDs on both MCUs will be turned off. The two PC serial terminals will go back to main menu shown below:

# Welcome to CECS 447 Project 2 – UART

# MCU1

# Main Menu

# PCMCU1 LED Control.

# MCU1MCU2 Color Wheel.

# PCMCU1MCU2 PC Chat Room

Please choose a communication mode (type 1 or 2 or 3 followed by a return):

# Welcome to CECS 447 Project 2 – UART

# MCU2

# Waiting for command from MCU1 …

PC Serial Terminal 1		                          PC Serial Terminal 2

# Mode 3-Chat Room: Upon entering this mode, the two PC terminals will display the following messages:

# Mode 3 MCU2: Chat Room

# Press sw1 at any time to exit the chat room.

# Waiting for a message from MCU1 …

# Mode 3 MCU1: Chat Room

# Press sw1 at any time to exit the chat room.

# Please type a message end with a return (less than 20 characters):

|

PC Serial Terminal 1		                          PC Serial Terminal 2

# MCU1 will be the one to initiate the chat.

# Step 1: As soon as a user enters a message in PC serial terminal 1, MCU1 will send the message to MCU2 and update its PC serial terminal display. After receiving the message sent from MCU1, MCU1 will be in a listen mode, MCU2 will update PC serial terminal 2 and get ready to receive any message entered on its PC serial terminal. Here is an example of the two terminals’ display:

# Mode 3 MCU1: Chat Room

# Press sw1 at any time to exit the chat room.

# Please type a message end with a return (less than 20 characters):

Hello MCU2!

|

# Mode 3 MCU2: Chat Room

# Press sw1 at any time to exit the chat room.

# Waiting for a message from MCU1 …

# Hello MCU2!

# |

PC Serial Terminal 1		                          PC Serial Terminal 2

# Step 2: As soon as a user enters a message in PC serial terminal 2, MCU2 will send the message to MCU1 and update its PC serial terminal display. After receiving the message sent from MCU2, MCU2 will be in a listen mode, MCU1 will update its PC serial terminal and get ready to receive any message entered on its PC serial terminal. Here is an example of the two terminals’ display:

# Mode 3 MCU1: Chat Room

# Press sw1 at any time to exit the chat room.

# Please type a message end with a return (less than 20 characters):

Hello MCU2!

# Hello MCU1!

# |

# Mode 3 MCU2: Chat Room

# Press sw1 at any time to exit the chat room.

# Waiting for a message from MCU1 …

# Hello MCU2!

# Hello MCU1!

# |

PC Serial Terminal 1		                          PC Serial Terminal 2

# Step 1 – Step 2 will be repeated until any one of the two MCUs’ sw1 is pressed. When this event is detected, both MCU will exit mode 3 and the two serial terminals’ display will go back to main menu shown below:

# Welcome to CECS 447 Project 2 – UART

# MCU1

# Main Menu

# PCMCU1 LED Control.

# MCU1MCU2 Color Wheel.

# PCMCU1MCU2 PC Chat Room

Please choose a communication mode (type 1 or 2 or 3 followed by a return):

# Welcome to CECS 447 Project 2 – UART

# MCU2

# Waiting for command from MCU1 …

PC Serial Terminal 1		                          PC Serial Terminal 2

How to set up serial terminal: