# CECS 447 – Project 1: Digital Piano

Total: 30 points (+2 extra credit)

# Overview

In this project you will build two embedded systems (two implementations of the same digital piano features) using the TM4C123 LaunchPad and external push buttons/speaker hardware. Both systems support the same user-facing behavior, but generate audio differently:

- System P1 (Digital Output): outputs a square wave to drive the speaker.

- System P2 (Analog Output): outputs a sine wave using a 6-bit R/2R DAC and a 64-sample sinusoid wavetable.

Each system has two modes:

- Piano Mode: press-and-hold one of 4 piano keys to play a note continuously; release stops the note. Only one key is assumed pressed at a time.

- Auto-Play Mode: plays one of three songs; the selected song repeats until changed.

# Learning objectives

By the end of this project, you should be able to:

Configure and use GPIO pins for input (push buttons) and output (audio).

Use a hardware timer interrupt to generate precise periodic signals.

Use GPIO interrupts for responsive user input handling.

Design and build a simple 6‑bit R‑2R DAC and drive it from the MCU.

Play notes and simple songs by controlling tone frequency and note duration.

Learn how to generate sound with both digital signals and analog signals.

# Hardware/Components

- You will use: TM4C123 LaunchPad, 4 push buttons (piano keys; optional 10kΩ resistors), speaker + audio amplifier, and resistors to build a 6-bit R/2R DAC (System P2).

Reference example projects: Music, DAC_3bit.

# System requirements

You will build two systems that implement the same user‑visible behavior but differ in how audio is generated:

| System | Audio output |
| --- | --- |
| System 1 | Digital square wave directly driven from the MCU to the speaker. |
| System 2 | Analog sine wave produced by a 6‑bit R2R DAC driven by the MCU and going to the amplifier/speaker. |

# User controls

Assume that at any time no more than one piano key is pressed.

| Control | Action | Behavior |
| --- | --- | --- |
| 4 piano keys (push buttons) | Press and hold | Play the assigned note continuously while held; stop immediately on release. |
| Switch 1 (left switch) | Press | Toggle between Piano mode and Auto‑play mode. |
| Switch 2 (right switch) | Press | In Piano mode: change octave. In Auto‑play mode: change song. |

# Operating Modes

## Mode 1 – Piano mode (default on reset)

In Piano mode, each push button acts as a piano key. When a key is pressed and held, output the corresponding tone. The tone must continue without gaps while the button remains pressed and must stop when the button is released.

Octave selection: Switch 2 cycles through at least three octaves in round‑robin order: Lower C, Middle C, Upper C. The system starts in Lower C after reset.

### Note Frequencies

| Note | Lower C (Hz) | Middle C (Hz) | Upper C (Hz) |
| --- | --- | --- | --- |
| C | 262 | 523 | 1046 |
| D | 294 | 587 | 1174 |
| E | 330 | 659 | 1318 |
| F | 349 | 698 | 1396 |

The octave selected in Piano mode will be used as the starting octave when entering Auto‑play mode.

## Mode 2 – Auto‑play mode

In Auto‑play mode, the system automatically plays one of the following songs:

Mary Had a Little Lamb

Twinkle Twinkle Little Star

Happy Birthday

On entry to Auto‑play mode, start playing the currently selected song at the octave previously chosen in Piano mode. While a song is playing, Switch 2 cycles to the next song (round‑robin). When the song selection changes, the newly selected song must restart from its beginning immediately. Song switching may occur at any time during playback.

# Implementation constraints

System clock must be greater than 16 MHz and less than 80 MHz.

You must use interrupts for: (1) a hardware timer and (2) the push buttons/switches.

You may choose any hardware timer(s) for timing control.

# Waveform generation details

System 1 (square wave): Use a hardware timer interrupt to toggle a GPIO output at the correct rate for the selected note frequency.

System 2 (sine wave via DAC): Build a 6‑bit R‑2R ladder DAC and output a sampled sinusoid. Use a 64‑sample sine lookup table for one cycle. The timer interrupt should advance the sample index at a fixed sample rate so that the output frequency matches the desired note.

# Extra credit (2 points)

Add a short video‑game song as a fourth option in Auto‑play mode.

# Appendix: Song note sequences

The following note and duration sequences may be used in your project. Your implementation may represent notes and durations in any convenient data structure. Duration table: 4 for a quarter note.

| Song | Note Table | Duration TAble |
| --- | --- | --- |
| Mary Had a Little Lamb | ¦E D C D¦ E E E-¦ ¦D D D-¦ E G G-¦ ¦E D C D ¦ E E E -¦ ¦D D E D C - ¦ | ¦4, 4, 4, 4, 4, 4, 8¦ ¦4, 4, 8, 4, 4, 8¦ ¦ 4, 4, 4, 4, 4, 4, 8¦ ¦ 4, 4, 4, 4, 8¦ |
| Twinkle Twinkle Little Star | ¦C C G G¦A A G -¦F F E E¦D D C -¦ ¦G G F F¦E E D -¦G G F F¦E E D -¦ ¦C C G G¦A A G -¦F F E E¦D D C -¦ | ¦4,4,4,4¦4,4,8¦4,4,4,4¦4,4,8¦ ¦4,4,4,4¦4,4,8¦4,4,4,4¦4,4,8¦ ¦4,4,4,4¦4,4,8¦4,4,4,4¦4,4,8¦ |
| ## Happy Birthday | ¦C C¦D C F¦E C C¦D C G¦F C C¦ ¦C’ A F¦E D B B¦A F G¦F¦ | ¦3,1¦4,4,4¦8,3,1¦4,4,4¦8,3,1¦ ¦4,4,4¦4,4,3,1¦4,4,4¦12¦ |